Day 7 — RTL Specification Materials

Sequence detector interface, traffic-light state diagram, corner cases for all three RTLs, and the FIFO overview.

# Item 3 — Interface Table: RTL #1 (Sequence Detector)

Every port, every column filled, per Learning Topic 7.3’s requirement that a description state when a signal is asserted and what it causes — not just restate the port name.

| Port | Direction | Width | Clock Domain | Active Level | Description |
| --- | --- | --- | --- | --- | --- |
| clk | input | 1 | — (source) | rising-edge | System clock. All sequential logic in this design (state register, output register) updates on the rising edge of clk. |
| rst_n | input | 1 | clk | active-low | Asynchronous reset. When LOW, forces the FSM to IDLE and detected to 0, independent of clk activity. |
| din | input | 1 | clk | active-high, sampled | Serial data input bit, sampled once per rising edge of clk. The FSM tracks how much of the target sequence 1011 has been matched using this bit. |
| detected | output | 1 | clk | active-high, one-cycle pulse | Pulses HIGH for exactly one clock cycle when the FSM confirms din has completed an occurrence of 1011 (overlapping detection — see Day 2/4). It is a Moore output: a pure combinational decode of the current state (detected = (state == S1011)), HIGH only while the FSM sits in the dedicated DETECT state S1011, with no dependency on din. Because S1011 is entered the cycle after the 4th matching bit is sampled, the pulse is one clock of ordinary FSM latency after that bit — confirmed via simulation in experiments/seq_detect_1011_smoke_log.md (pulse in cycle 8, in state S1011). |

# Item 4 — ASCII State Diagram: Traffic Light Controller

+--------------------------------------------------------------+

|  reset --> MAIN_GREEN, counter loaded with MAIN_GREEN_TIME-1   |

|  (same as every reload; first MAIN_GREEN is a full 4 cycles)   |

+---------------+----------------------------------------------+

v

count!=0            count!=0             count!=0             count!=0

+----+               +----+               +----+               +----+

|    v               |    v               |    v               |    v

+---------------+    +---------------+    +---------------+    +---------------+

-->|  MAIN_GREEN   |--->|  MAIN_YELLOW  |--->|  SIDE_GREEN   |--->|  SIDE_YELLOW  |--+

|  |  (4 cycles)   |    |  (2 cycles)   |    |  (6 cycles)   |    |  (2 cycles)   |  |

|  +---------------+    +---------------+    +---------------+    +---------------+  |

|       count==0             count==0             count==0             count==0      |

+------------------------------------------------------------------------------------+

count==0 (wrap back to MAIN_GREEN)

Notes

- Every self-loop (count!=0) means “stay in this state, the down-counter just decremented.” Every forward arrow (count==0) means “this phase’s duration just expired, advance to the next phase in the fixed sequence.”

- There is no external input anywhere in this diagram — the only thing that ever drives a transition is the counter reaching zero (Learning Topic 5.1’s FSM+datapath pattern).

- Reset forces MAIN_GREEN and loads the counter with MAIN_GREEN_TIME - 1 — exactly the same value a normal reload uses — so the first MAIN_GREEN phase lasts a full MAIN_GREEN_TIME (4) cycles, identical to every later MAIN_GREEN. The first draft loaded MAIN_GREEN_TIME (not -1) at reset, which made the first phase 5 cycles; see ISSUE-D5-02 (corrected). A duration of D cycles corresponds to the counter stepping D-1, D-2, ... , 0, so the reload value is always D-1, reset included.

# Item 2 — Corner Cases: All Three RTLs

At least 3 per design, per the assignment (I’ve listed 4 each for margin).

## RTL #1 — Sequence Detector

| # | Corner Case | Expected Behavior |
| --- | --- | --- |
| 1 | Overlapping match immediately after a previous match (e.g., ...1011011...) | Both occurrences of 1011 must be detected. Confirmed by the dedicated self-checking overlap regression (seq_detect_1011_overlap_tb) driving 1011011 — two detections fire, in cycles 5 and 8 (3-cycle gap, the minimum for overlapping 1011). Note: the Day-4 smoke vector 1101011 does not exercise this — it contains only one 1011 — which is why a dedicated overlap test exists. |
| 2 | Reset asserted mid-sequence (e.g., FSM is in S101 when rst_n drops) | FSM must return cleanly to IDLE, detected must go LOW, and any partial match progress must be discarded — not carried across reset. |
| 3 | din held constant (all 0s or all 1s) for many cycles | Must never falsely assert detected; must correctly self-loop (IDLE on all-0s, S1 on all-1s) indefinitely without erroring. |
| 4 | First bit after reset is a 1 | FSM must immediately begin progressing from IDLE toward S1 — no “warm-up” cycle required before the FSM starts tracking progress. |

## RTL #2 — Traffic Light Controller

| # | Corner Case | Expected Behavior |
| --- | --- | --- |
| 1 | Reset asserted while mid-countdown in a non-initial state (e.g., reset hits during SIDE_GREEN) | Must always return to MAIN_GREEN and reload the counter with MAIN_GREEN_TIME - 1 (the same value a normal reload uses), giving a full 4-cycle MAIN_GREEN regardless of which state or count value reset happened to catch the FSM at. |
| 2 | Wraparound from SIDE_YELLOW back to MAIN_GREEN | Must reload for a full MAIN_GREEN_TIME duration every time this state is re-entered — not just the first time. The 20-cycle smoke test (one full 14-cycle loop plus 6 cycles into the second) now shows the first and second MAIN_GREEN both lasting a true 4 cycles, verifiable from the exposed count column. Note: it was the corrected logging (honest per-cycle sampling + count column, ISSUE-D5-03) that made ISSUE-D5-02 visible — the earlier negedge-loop sampling had masked it by dropping the first cycle from the printout. |
| 3 | A duration parameter set to the minimum realistic value (1 cycle) | The load-vs-decrement priority (count == 0 check) must still advance the FSM exactly one cycle later, without skipping the state or holding an extra cycle. Confirmed by the min-duration regression test (traffic_light_min_duration_tb, MAIN_YELLOW_TIME=1), which shows MAIN_YELLOW occupying exactly one cycle (count loaded to 0, reloads next edge) between a 4-cycle green and a 6-cycle green — closing ISSUE-D7-01. |
| 4 | Two adjacent states sharing the same duration value | The counter reaching zero must still trigger a transition to the correct next state (per the fixed sequence), not “confuse” which state comes next just because two durations happen to coincide numerically. |

## RTL #3 — Synchronous FIFO

| # | Corner Case | Expected Behavior |
| --- | --- | --- |
| 1 | Write while full | Blocked — no data corruption, count never exceeds DEPTH. Confirmed in Day 6 smoke test (poison value 0xFF never appeared in any later read). |
| 2 | Read while empty | Blocked — no count underflow. Confirmed in Day 6 smoke test. |
| 3 | Simultaneous read+write, in each of the full / empty / partially-filled sub-cases | count behaves correctly in all three: unchanged when partially filled, decrements-only when full, increments-only when empty. All three confirmed via the extended Day 6 testbench (ISSUE-D6-01). |
| 4 | Pointer wraparound exactly at the DEPTH boundary | Writing/reading across the “seam” where wr_ptr or rd_ptr wraps from DEPTH-1 back to 0 must not skip or duplicate a memory slot. Confirmed by the self-checking multi-wrap regression (sync_fifo_multiwrap_tb, ISSUE-D7-02): 3 consecutive fill-then-drain cycles (24 writes / 24 reads, both pointers wrapping multiple times) with every read matched against a software reference queue — PASS. |

# Item 5 — One-Paragraph FIFO Overview

For a DV engineer who has never seen it.

This is a standard single-clock-domain synchronous FIFO with 8 entries and 8 bits of width (both parameterizable), featuring write-side control (wr_en, din), read-side control (rd_en, dout), and full and empty status flags; internally it comprises an 8-entry memory array, write and read pointers, and a count register that directly controls the status flags since comparing pointers alone cannot distinguish between the completely-empty and completely-full states (both make the pointers equal). Writes are blocked when the FIFO is full and reads when empty, but the design correctly handles simultaneous wr_en and rd_en assertions in the same cycle — both proceed without changing the entry count, except in edge cases where the FIFO is already completely full (only the read executes, freeing a slot) or completely empty (only the write executes). Note that dout is a combinational “peek” at the read pointer’s target rather than a registered output, updating immediately; when empty=1, it displays stale leftover data rather than meaningful content, so any test reading dout must gate the check on empty first.