Regression Test Log

ISSUE-D7-01  —  Traffic Light Minimum Duration

# Purpose

Confirm the load-vs-decrement priority logic correctly handles a state duration of 1 cycle — the minimum realistic value — without skipping the state or hanging for an extra cycle.

# Test

tb/traffic_light_min_duration_tb.sv instantiates traffic_light with MAIN_YELLOW_TIME(1) overridden (other durations left at default: 4, 6, 2). One clean pass through the 1-cycle phase fully answers the question; the run continues 16 cycles into the following phases for extra confidence.

Logging uses the corrected honest per-cycle scheme (ISSUE-D5-03) and exposes the internal count, so durations are read numerically, not by counting rows.

# Result

Verbatim — corrected RTL + corrected testbench.

VCD info: dumpfile dump.vcd opened for output.

time    cycle   state           main_r  main_y  main_g  side_r  side_y  side_g  count

25000   1       MAIN_GREEN      0       0       1       1       0       0       3

35000   2       MAIN_GREEN      0       0       1       1       0       0       2

45000   3       MAIN_GREEN      0       0       1       1       0       0       1

55000   4       MAIN_GREEN      0       0       1       1       0       0       0

65000   5       MAIN_YELLOW     0       1       0       1       0       0       0

75000   6       SIDE_GREEN      1       0       0       0       0       1       5

85000   7       SIDE_GREEN      1       0       0       0       0       1       4

95000   8       SIDE_GREEN      1       0       0       0       0       1       3

105000  9       SIDE_GREEN      1       0       0       0       0       1       2

115000  10      SIDE_GREEN      1       0       0       0       0       1       1

125000  11      SIDE_GREEN      1       0       0       0       0       1       0

135000  12      SIDE_YELLOW     1       0       0       0       1       0       1

145000  13      SIDE_YELLOW     1       0       0       0       1       0       0

155000  14      MAIN_GREEN      0       0       1       1       0       0       3

165000  15      MAIN_GREEN      0       0       1       1       0       0       2

175000  16      MAIN_GREEN      0       0       1       1       0       0       1

Min-duration regression test complete.

Day-05_03_traffic_light_min_duration_tb.sv:58: $finish called at 175000 (1ps)

Shaded rows are the YELLOW phases. Cycle 5 — the overridden 1-cycle MAIN_YELLOW — is the row this test exists to check.

# Verdict

MAIN_YELLOW occupies exactly one cycle (cycle 5, count=0) between a 4-cycle MAIN_GREEN (count 3→0) and a 6-cycle SIDE_GREEN (count 5→0): the FSM enters it, holds for exactly the overridden duration, and advances on the next edge. No skip, no extra hang cycle.

A duration-1 phase is loaded with count=0 and is immediately due for reload the next cycle — the load-vs-decrement priority (count == 0 check taking priority over plain decrement) generalizes correctly to the smallest possible duration.

Incidental confirmation: this run also shows the reset fix from ISSUE-D5-02 — the first MAIN_GREEN is a true 4 cycles (count 3, 2, 1, 0), matching the corrected reset load of MAIN_GREEN_TIME-1.

ISSUE-D7-01 is resolved.