Day 4 Smoke Test Log

seq_detect_1011

Simulator used: Icarus Verilog v12.0 (iverilog -g2012), run locally.

# 1.  Design under test

A true 5-state Moore FSM: states IDLE, S1, S10, S101, and a dedicated DETECT state S1011. detected is a pure function of state (detected = (state == S1011)), so it depends on no input directly.

# 2.  Compile command

iverilog -g2012 -o sim_d4_smoke.vvp Day-04_01_seq_detect_1011.sv \

Day-04_02_seq_detect_1011_smoke_tb.sv

Compiled clean — zero errors, zero warnings.

# 3.  Run command

vvp sim_d4_smoke.vvp

# 4.  Terminal output

Each row shows the state the FSM is in during that cycle (pre-edge), the din bit consumed on that cycle’s rising edge, and detected for that cycle.

VCD info: dumpfile dump.vcd opened for output.

time    cycle   state   din     detected

25000   1       IDLE    1       0

35000   2       S1      1       0

45000   3       S1      0       0

55000   4       S10     1       0

65000   5       S101    0       0

75000   6       S10     1       0

85000   7       S101    1       0

95000   8       S1011   0       1

105000  9       S10     0       0

Smoke test complete.

Day-04_02_seq_detect_1011_smoke_tb.sv:96: $finish called at 105000 (1ps)

# 5.  Result vs. prediction

Stimulus 1 1 0 1 0 1 1 (then trailing 0s). The pattern 1011 is completed by the 4th matching bit — the 7th input bit. Prediction from the Day 2 hand-trace, confirmed exactly:

| Cycle | State (this cycle) | din consumed | detected | Note |
| --- | --- | --- | --- | --- |
| 4 | S10 | 1 | 0 | building the match |
| 5 | S101 | 0 | 0 | fell back to S10 next |
| 6 | S10 | 1 | 0 | — |
| 7 | S101 | 1 | 0 | 4th bit sampled here → FSM moves to S1011 |
| 8 | S1011 | 0 | 1 | DETECT state → detected pulses HIGH |
| 9 | S10 | 0 | 0 | pulse clears after exactly one cycle |

detected is a clean one-cycle pulse, HIGH only in cycle 8, and it occurs one clock after the completing 4th bit (applied in cycle 7 while the FSM is still in S101). This is the ordinary registered latency of a Moore FSM output — the pulse is in the DETECT state, not in S101, which is exactly why a genuine Moore detector needs the 5th state.