Day 4 Smoke Test Log

seq_detect_1011

Simulator used: Icarus Verilog v12.0 (iverilog -g2012), run locally.

# 1.  Design under test

A true 5-state Moore FSM: states IDLE, S1, S10, S101, and a dedicated DETECT state S1011. detected is a pure function of state (detected = (state == S1011)), so it depends on no input directly.

# 2.  Compile command

iverilog -g2012 -o sim_d4_overlap.vvp Day-04_01_seq_detect_1011.sv \

Day-04_04_seq_detect_1011_overlap_tb.sv

Compiled clean — zero errors, zero warnings.

# 3.  Run command

vvp sim_d4_overlap.vvp

# 4.  Terminal output

Each row shows the state the FSM is in during that cycle (pre-edge), the din bit consumed on that cycle’s rising edge, and detected for that cycle.

VCD info: dumpfile dump.vcd opened for output.

time    cycle   state   din     detected

25000   1       IDLE    1       0

35000   2       S1      0       0

45000   3       S10     1       0

55000   4       S101    1       0

65000   5       S1011   0       1

75000   6       S10     1       0

85000   7       S101    1       0

95000   8       S1011   0       1

105000  9       S10     0       0

PASS: overlapping 1011 detected twice (cycles 5 and 8, 3-cycle gap).

Day-04_04_seq_detect_1011_overlap_tb.sv:84: $finish called at 105000 (1ps)