Day 5 Smoke Test Log

traffic_light

Simulator used: Icarus Verilog v12.0 (iverilog -g2012), run locally.

# 1.  Compile command

iverilog -g2012 -o sim_d5_smoke.vvp Day-05_01_traffic_light.sv \

Day-05_02_traffic_light_smoke_tb.sv

Compiled clean — zero errors, zero warnings.

# 2.  Run command

vvp sim_d5_smoke.vvp

# 3.  Terminal output (verbatim — corrected RTL + corrected testbench)

The rightmost count column is the internal down-counter, exposed so each phase’s duration is directly verifiable (a phase of duration D shows the counter stepping D-1, D-2, … , 0 — i.e. D rows).

VCD info: dumpfile dump.vcd opened for output.

time    cycle   state           main_r  main_y  main_g  side_r  side_y  side_g  count

25000   1       MAIN_GREEN      0       0       1       1       0       0       3

35000   2       MAIN_GREEN      0       0       1       1       0       0       2

45000   3       MAIN_GREEN      0       0       1       1       0       0       1

55000   4       MAIN_GREEN      0       0       1       1       0       0       0

65000   5       MAIN_YELLOW     0       1       0       1       0       0       1

75000   6       MAIN_YELLOW     0       1       0       1       0       0       0

85000   7       SIDE_GREEN      1       0       0       0       0       1       5

95000   8       SIDE_GREEN      1       0       0       0       0       1       4

105000  9       SIDE_GREEN      1       0       0       0       0       1       3

115000  10      SIDE_GREEN      1       0       0       0       0       1       2

125000  11      SIDE_GREEN      1       0       0       0       0       1       1

135000  12      SIDE_GREEN      1       0       0       0       0       1       0

145000  13      SIDE_YELLOW     1       0       0       0       1       0       1

155000  14      SIDE_YELLOW     1       0       0       0       1       0       0

165000  15      MAIN_GREEN      0       0       1       1       0       0       3

175000  16      MAIN_GREEN      0       0       1       1       0       0       2

185000  17      MAIN_GREEN      0       0       1       1       0       0       1

195000  18      MAIN_GREEN      0       0       1       1       0       0       0

205000  19      MAIN_YELLOW     0       1       0       1       0       0       1

215000  20      MAIN_YELLOW     0       1       0       1       0       0       0

Smoke test complete.

Day-05_02_traffic_light_smoke_tb.sv:71: $finish called at 215000 (1ps)

# 4.  Result vs. prediction — full cycle verified

| Phase | Predicted duration | Actual (cycles occupied) | Actual duration | Match? |
| --- | --- | --- | --- | --- |
| MAIN_GREEN (1st) | 4 | 1–4 (count 3→0) | 4 | ✓ |
| MAIN_YELLOW (1st) | 2 | 5–6 (count 1→0) | 2 | ✓ |
| SIDE_GREEN | 6 | 7–12 (count 5→0) | 6 | ✓ |
| SIDE_YELLOW | 2 | 13–14 (count 1→0) | 2 | ✓ |
| MAIN_GREEN (2nd) | 4 | 15–18 (count 3→0) | 4 | ✓ |
| MAIN_YELLOW (2nd) | 2 | 19–20 (count 1→0, sim ends) | 2 | ✓ |

Full 14-cycle loop (4+2+6+2) confirmed, and the first MAIN_GREEN and the second (post-wraparound) MAIN_GREEN are now both a true 4 cycles, provable from the count column (3→0 each time). All six lights are one-hot-per-road every cycle — never both roads non-red, never two colours on one road.

# 5.  Two bugs fixed to get here

Honest account — supersedes an earlier, wrong write-up.

An earlier version of this log reported “4 cycles ✓” and claimed the first MAIN_GREEN “was 3 cycles before the fix.” That was wrong, and it hid a real bug. What was actually happening:

- ISSUE-D5-02 (RTL off-by-one). The reset branch loaded count <= MAIN_GREEN_TIME (= 4) instead of MAIN_GREEN_TIME - 1 (= 3). Because every other phase reloads with duration - 1, this made the very first MAIN_GREEN last 5 cycles while every later MAIN_GREEN lasted 4. Fix: load MAIN_GREEN_TIME - 1 at reset, matching the reload path.

- ISSUE-D5-03 (testbench sampling artifact that masked the above). The original TB deasserted reset after two @(negedge clk) waits, then logged inside a for loop of @(negedge clk). The first logged row therefore landed after the first active clock edge, so the reset-loaded cycle was never printed. The first MAIN_GREEN (physically 5 cycles) printed as 4 rows, and every later MAIN_GREEN (physically 4) also printed as 4 — so the two looked identical and the off-by-one was invisible. Worse, the same TB run against the corrected RTL prints only 3 MAIN_GREEN rows, which would look like a failure. Fix: log in an always @(posedge clk) block that reads the registered values in the cycle they are held, so cycle 1 is the first cycle out of reset (reset-loaded count included), and add the count column so durations are self-evident.

Lesson logged for the KB: a smoke test that only prints and is read “by eye” can mask a bug if its sampling is misaligned — the second-loop “consistency” check did not catch the bug, it hid it, because both phases were mis-sampled the same way. The count column removes the ambiguity.

# 6.  Min-duration regression (traffic_light_min_duration_tb, MAIN_YELLOW_TIME = 1)

Closes ISSUE-D7-01: confirms a duration-1 phase behaves correctly (loaded with count=0, so it occupies exactly one cycle and reloads the next). Verbatim:

VCD info: dumpfile dump.vcd opened for output.

time    cycle   state           main_r  main_y  main_g  side_r  side_y  side_g  count

25000   1       MAIN_GREEN      0       0       1       1       0       0       3

35000   2       MAIN_GREEN      0       0       1       1       0       0       2

45000   3       MAIN_GREEN      0       0       1       1       0       0       1

55000   4       MAIN_GREEN      0       0       1       1       0       0       0

65000   5       MAIN_YELLOW     0       1       0       1       0       0       0

75000   6       SIDE_GREEN      1       0       0       0       0       1       5

85000   7       SIDE_GREEN      1       0       0       0       0       1       4

95000   8       SIDE_GREEN      1       0       0       0       0       1       3

105000  9       SIDE_GREEN      1       0       0       0       0       1       2

115000  10      SIDE_GREEN      1       0       0       0       0       1       1

125000  11      SIDE_GREEN      1       0       0       0       0       1       0

135000  12      SIDE_YELLOW     1       0       0       0       1       0       1

145000  13      SIDE_YELLOW     1       0       0       0       1       0       0

155000  14      MAIN_GREEN      0       0       1       1       0       0       3

165000  15      MAIN_GREEN      0       0       1       1       0       0       2

175000  16      MAIN_GREEN      0       0       1       1       0       0       1

Min-duration regression test complete.

Day-05_03_traffic_light_min_duration_tb.sv:58: $finish called at 175000 (1ps)

MAIN_YELLOW occupies exactly one cycle (cycle 5, count = 0) between a 4-cycle MAIN_GREEN and a 6-cycle SIDE_GREEN — no skip, no extra hang cycle.